A Discrete Event System Simulation of the Mercedes AMG Petronas F1 Team at the Austrian Grand Prix

CSCI B450

Jarrett Kizer

December 8, 2023

The Austrian Grand Prix in 2023 consisted of 20 drivers for 10 teams racing 71 laps around the circuit. In Formula 1, each team must create a car that meets regulations set by the FIA. So the performances of the cars will vary. To produce a more accurate simulation, a single team was chosen to simulate. The goal of this simulation would be to compare two different tire strategies and determine which (if either) was better. Three dry tire compounds are available to use during a given race, a soft compound, a medium compound, and a hard compound. During a dry race, a driver must use at least two different tire compounds. For this simulation, it is assumed that the same weather, temperature, and track conditions as those present from the real-world race that the data was collected from.

Each trial for this simulation was modeled as a race at the Austrian Grand Prix. This means it consisted of 71 lap times, and the total time, minus any time added from safety cars, would be the output of each trial. For the experiments, 400 trials were run for both hypothesized parameters. Each lap time consisted of several time factors which we will explore.

The first one which all the following data was adjusted for was the car weight from fuel usage. Formula 1 cars are not allowed to refuel during a race, due to concerns of it being a fire hazard. According to Vyssion and jjn9128, from F1Technical.net, every Kilogram of weight adds an average of 0.03 seconds to the lap time. Also, according to Stefan Kristensen from motorsportexplained.com, F1 cars get an average of 7.1 mpg, for our purposes, this was converted to km/kg, then to kg/lap. The final model which was used is:

Fuel Weight Time = 0.03 (Starting Fuel Weight – lap Number * kg/lap)

The largest component of individual lap times is the tire models. These were created from the real lap times, acquired from the Formula 1 World Championship (1950-2023) dataset on Kaggle.com. All the lap times were adjusted according to the fuel model, to compensate for when during the race the time was collected. Seen below in Figure 1 and Figure 2 are the graphs created for the medium and hard compound tires. The soft tire was not used on race day, however, it was used in qualifying, so see the adjusted model for soft tires in Figure 3. In these models, x is the tire age, and y is the time from the tire compound.

Figure 1.

Figure 2.

Figure 3.

The residuals of these are seen in Figure 4 and Figure 5. These fit a Weibull distribution. The reason for the variation in lap time is caused by DRS, and driver error, for our purposes we will assume that the Weibull distribution will account for these variances. Due to the lack of data for the soft tire, it is assumed that it follows a similar process as the medium and hard tire and that given more data a Weibull distribution would have fit those residuals. The parameters from these calculations can be seen in data table 1. An extra parameter, offset was used, since the Weibull CDF cannot produce negative values. The values were generated and then shifted in the negative direction by the offset amount. (ignore the p value for now).

| data table 1 | alpha | beta | offset | p-value |
| --- | --- | --- | --- | --- |
| soft | 1.063 | 8.146 | 1 | N/A |
| medium | 1.1247 | 3.1788 | 1 | 0.049879 |
| hard | 2.0908 | 6.1714 | 2 | 0.112339 |

Figure 4

Figure 5

The next factor that influenced lap times was the time from pitting. To change the tires the driver must pit. So according to programmed logic, if it is decided that the driver should pit, then time is added to the total lap time. These times are different depending on when it happens. That is does it happen under a safety car, a virtual safety car, or no safety car? The process for when these happen will be discussed later, but it was determined that when it occurs, it follows a lognormal distribution, as seen in figure 6, figure 7, and figure 8. The parameters for these 3 types can be seen in data table 2, along with the p-value calculated from a Student T-Test. All distributions will be evaluated at a significance level of 5%, which leads to a critical value of 1.645. Thus this test fails to reject all 3 distributions.

| data table 2 | mean | St.dev | p-value |
| --- | --- | --- | --- |
| full | 2.896599 | 0.171157 | 0.178585 |
| virt | 3.122989 | 0.059309 | 0.134397 |
| none | 3.16003 | 0.132576 | 0.08467 |

Figure 6

Figure 7

Figure 8

The final major factor was safety cars. A Safety car is deployed whenever a crash occurs. To determine if a crash occurred during the simulation, the last 8 Formula 1 races were evaluated for the number of crashes over 71 laps, then the average of this was taken and a discrete cdf was used to determine whether a crash occurred. The value found was 0.020344. There was a rule change in 2021, which increased the amount of virtual safety cars deployed. So if a crash occurs it follows a binomial distribution with a 0.4 chance of a full safety car and a 0.6 chance of a virtual safety car. The duration of these was determined using the CDF’s shown in data table 3 and data table 4. During either of these, tires age at a rate of 0.5 per lap instead of a rate of 1 per lap.

| Data Table 3 – virtual |  |  |
| --- | --- | --- |
| virtual | prob | CDF |
| 1 | 0.25 | 0.25 |
| 2 | 0.25 | 0.5 |
| 3 | 0.5 | 1 |

| Data Table 4 - full |  |  |
| --- | --- | --- |
| full | prob | CDF |
| 3 | 0.4 | 0.4 |
| 4 | 0.2 | 0.6 |
| 5 | 0.2 | 0.8 |
| 6 | 0.2 | 1 |

Time added from the two types of safety cars was initially included in the model, however, this was very large, and dominated the results of the simulation, making it difficult to determine the effect of the different strategies, so for the final simulation, the additional time was removed. For full safety cars, they followed a model, with residuals generated using a normal distribution. For virtual safety cars, it was modeled using a normal distribution.

For validation purposes, it was necessary to generate time added to the laps from full or virtual safety cars. However, this was not added to the final simulation of lap times, since this added time would dominate the results and make it difficult to determine which strategy performed better. For time from the virtual safety car, it was determined to follow a normal distribution with the parameters listed in data table 5. For the full safety car, a model was created (figure 9) and the following was used as a rule for the added time. The first lap of a safety car used the 1 as x. The final lap of a safety car used 3 as x, and the rest of the laps used 2 as x.

.

|  | mean | St.dev | coef 1 | coef 2 | coef 3 |
| --- | --- | --- | --- | --- | --- |
| virtual | 11.76168 | 8.398172 | N/A | N/A | N/A |
| full | 0.001928 | 8.936815 | -21.62 | 87.151 | -43.41 |

Figure 9

To validate the data it was assumed, that if a collection of lap times, generated from the above methods was from the same distribution of the real lap times provided by the data set, then the simulation is valid. The confidence level was determined to be 5%. To do this, seven trials were run and all 71 lap times from each distribution were documented. After this a Students T Test found a P value of 0.1512, using the above confidence level would provide a critical value of 1.645, thus, we fail to reject the null hypothesis. That being we failed to say that given the evidence, we failed to say the real data and the generated data came from different distribution methods.

For the simulation, the null hypothesis was that strategy 1 and strategy 2 did not provide any significant performance difference. The alternate hypothesis was that strategy 2 provided a significant performance increase over strategy 1.

H0: μ1=μ2

H1: μ1>μ2

Below are strategy 1 and strategy 2.

Data table 5

Data table 6

If a safety car is deployed (full or virtual) it will cause the driver to pit early if it is a certain number of laps within its target pit time. If it is in the correct range, then it will cause the driver to switch from the current tire plan to the tire plan below it. The experiment used the strategies and ran 400 trials (each trial is equivalent to one race, so 71 laps) with an input seed of 12345. The results of both experiments are summarized numerically in data tables 7 and 8, and visually in figures 9 and 10.

Data table 7 and data table 8

Figure 9

Figure 10

To test the null and alternate hypotheses, these results were fed into a 2 sample T-Test, which produced the following values:

T: 13.5598

P-value: 3.8E-38

Using the same confidence level of 5% we find the p-value to be lower than 0.05, so reject the null hypothesis. That is we do not believe the strategies are equivalent, we believe strategy 2 is better than strategy 1.

Below are some improvements that could be made to generate a more accurate simulation, as well as some ideas for different hypotheses.

- Keep track of the position of all 20 drivers on the track, and how the proximity of the cars affects the lap times.

- Collect more data to determine how different track conditions and weather affect the lap times of the different tire compounds.

- You could create the tire distribution for a different team, and compare the results of the cars on the same strategy to determine which team performs better.

In conclusion, this experiment showed that strategy 2 was better than strategy 1. This simulation accomplished its goal of providing a statistically accurate representation of the Mercedes AMG Petronas F1 team at the 2023 Austrian Grand Prix and is built to possibly accommodate other teams or other circuits.

References

Vyssion and jjn9128. (2018, May). Hurry Up and Weight. F1technical.net.

https://www.f1technical.net/features/21637#:~:text=How%20this%20works%20out%20in,drag%20saved%20worth%20six%20hundredths

Kristensen, S. (2022, October 2). What mpg does an F1 car do? Motorsport Explained.

https://motorsportexplained.com/mpg-f1-car/

Vopani. (2023, August 24). Formula 1 World Championship (1950 - 2023). Kaggle.

https://www.kaggle.com/datasets/rohanrao/formula-1-world-championship-1950-2020